型号作假

1. 需求说明

要求能够随时设置手机的型号、品牌、蓝牙名称、WIFI热点名称、MTP名称、PTP名称和Windows系统资源管理器中磁盘的名称。

2. 修改文件：

3. 型号作假设置应用

ModelSetting

4. 公共部分修改：

（1）在device/alibaba/xunhu_public/FeatureOption中添加如下代码：

XUNHU_QTY_MODEL_SETTING

（2）在device/alibaba/xunhu_public/FeatureOption.txt中添加如下代码：

[XUNHU_QTY_MODEL_SETTING]---->[客制化型号、品牌、BT、WIFI AP、MTP、PTP、USB、Device Manager Name]---->[默认关闭]

[XUNHU_QTY_CUSTOM_MODEL]---->[同时设置model、bt、wifi ap、usb、mtp、ptp、dm的默认名称为该宏的值，这个宏依赖于XUNHU_QTY_MODEL_SETTING，如果没有打开XUNHU_QTY_MODEL_SETTING，则表示只设置一次，不能再改变]---->[默认为no，即不设置]

[XUNHU_QTY_MODEL]---->[设置手机型号默认名称，这个宏依赖于XUNHU_QTY_MODEL_SETTING，如果没有打开XUNHU_QTY_MODEL_SETTING，则表示只设置一次，不能再改变]--->[默认为no，即不设置]

[XUNHU_QTY_BRAND]---->[设置手机品牌默认名称，这个宏依赖于XUNHU_QTY_MODEL_SETTING，如果没有打开XUNHU_QTY_MODEL_SETTING，则表示只设置一次，不能再改变]--->[默认为no，即不设置]

[XUNHU_QTY_BT]---->[设置蓝牙默认名称，这个宏依赖于XUNHU_QTY_MODEL_SETTING，如果没有打开XUNHU_QTY_MODEL_SETTING，则表示只设置一次，不能再改变]--->[默认为no，即不设置]

[XUNHU_QTY_WIFI_AP]---->[设置wifi热点默认名称，这个宏依赖于XUNHU_QTY_MODEL_SETTING，如果没有打开XUNHU_QTY_MODEL_SETTING，则表示只设置一次，不能再改变]--->[默认为no，即不设置]

[XUNHU_QTY_USB]---->[设置USB默认名称，这个宏依赖于XUNHU_QTY_MODEL_SETTING，如果没有打开XUNHU_QTY_MODEL_SETTING，则表示只设置一次，不能再改变]--->[默认为no，即不设置]

[XUNHU_QTY_MTP]---->[设置MTP默认名称，这个宏依赖于XUNHU_QTY_MODEL_SETTING，如果没有打开XUNHU_QTY_MODEL_SETTING，则表示只设置一次，不能再改变]--->[默认为no，即不设置]

[XUNHU_QTY_PTP]---->[设置PTP默认名称，这个宏依赖于XUNHU_QTY_MODEL_SETTING，如果没有打开XUNHU_QTY_MODEL_SETTING，则表示只设置一次，不能再改变]--->[默认为no，即不设置]

[XUNHU_QTY_DM]---->[设置Windows系统资源管理器中手机磁盘默认名称，这个宏依赖于XUNHU_QTY_MODEL_SETTING，如果没有打开XUNHU_QTY_MODEL_SETTING，则表示只设置一次，不能再改变]--->[默认为no，即不设置]

(3)在/device/alibaba/xunhu_public/ProjectConfig.mk文件中添加如下代码：

XUNHU_AUTO_ADD_GLOBAL_DEFINE_BY_NAME = … XUNHU_QTY_MODEL_SETTING

XUNHU_QTY_MODEL_SETTING=no

XUNHU_QTY_CUSTOM_MODEL=no

XUNHU_QTY_MODEL=no

XUNHU_QTY_BRAND=no

XUNHU_QTY_BT=no

XUNHU_QTY_WIFI_AP=no

XUNHU_QTY_USB=no

XUNHU_QTY_MTP=no

XUNHU_QTY_PTP=no

XUNHU_QTY_DM=no

(4)在device/alibaba/common/resources/custom/engineer.xml文件中添加如下代码：

<!--		Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty		-->

<!--		Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称		-->

<customer>

<commandId>*#8185#</commandId>

<secretCode>android_secret_code://8185</secretCode>

</customer>

<customer>

<commandId>*#8186#</commandId>

<secretCode>android_secret_code://8186</secretCode>

</customer>

<customer>

<commandId>*#352137#</commandId>

<secretCode>android_secret_code://352137</secretCode>

</customer>

<!--		Xunhu: end custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty		→

（5）在device/mediatek/build/build/libs/custom.mk文件中添加如下代码：

#Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

#Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称

ifneq ($(strip $(XUNHU_QTY_CUSTOM_MODEL)), no)

XUNHU_QTY_MODEL := $(strip $(XUNHU_QTY_CUSTOM_MODEL))

XUNHU_QTY_BT := $(strip $(XUNHU_QTY_CUSTOM_MODEL))

XUNHU_QTY_WIFI_AP := $(strip $(XUNHU_QTY_CUSTOM_MODEL))

XUNHU_QTY_USB := $(strip $(XUNHU_QTY_CUSTOM_MODEL))

XUNHU_QTY_MTP := $(strip $(XUNHU_QTY_CUSTOM_MODEL))

XUNHU_QTY_PTP := $(strip $(XUNHU_QTY_CUSTOM_MODEL))

XUNHU_QTY_DM := $(XUNHU_QTY_CUSTOM_MODEL)

endif

#&&}}

（6）在/device/mediatek/mt6735/device.mk文件中添加如下代码：

#Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

#Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称

ifeq ($(strip $(XUNHU_QTY_MODEL_SETTING)), yes)

PRODUCT_PACKAGES += ModelSetting

endif

ifneq ($(strip $(XUNHU_QTY_CUSTOM_MODEL)), no)

XUNHU_QTY_MODEL := $(strip $(XUNHU_QTY_CUSTOM_MODEL))

XUNHU_QTY_BT := $(strip $(XUNHU_QTY_CUSTOM_MODEL))

XUNHU_QTY_WIFI_AP := $(strip $(XUNHU_QTY_CUSTOM_MODEL))

XUNHU_QTY_USB := $(strip $(XUNHU_QTY_CUSTOM_MODEL))

XUNHU_QTY_MTP := $(strip $(XUNHU_QTY_CUSTOM_MODEL))

XUNHU_QTY_PTP := $(strip $(XUNHU_QTY_CUSTOM_MODEL))

XUNHU_QTY_DM := $(XUNHU_QTY_CUSTOM_MODEL)

endif

$(shell mkdir -p out/Autogen_out/custom_model)

$(shell echo "#ifdef XUNHU_QTY_USB" > out/Autogen_out/custom_model/teksun_custom_model.h)

$(shell echo "#undef XUNHU_QTY_USB" >> out/Autogen_out/custom_model/teksun_custom_model.h)

$(shell echo "#endif" >> out/Autogen_out/custom_model/teksun_custom_model.h)

$(shell echo "#ifdef XUNHU_QTY_DM" >> out/Autogen_out/custom_model/teksun_custom_model.h)

$(shell echo "#undef XUNHU_QTY_DM" >> out/Autogen_out/custom_model/teksun_custom_model.h)

$(shell echo "#endif" >> out/Autogen_out/custom_model/teksun_custom_model.h)

ifneq ($(strip $(XUNHU_QTY_USB)),no)

$(shell echo "#define XUNHU_QTY_USB \"$(strip $(XUNHU_QTY_USB))\"" >> out/Autogen_out/custom_model/teksun_custom_model.h)

endif

ifneq ($(strip $(XUNHU_QTY_DM)),no)

$(shell echo "#define XUNHU_QTY_DM \"$(strip $(XUNHU_QTY_DM))\"" >> out/Autogen_out/custom_model/teksun_custom_model.h)

endif

#&&}}

5. 客制化型号和品牌：

在system/core/init/property_service.cpp文件中更新作假型号值：

（1）获取NV中的型号名称：

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(更新型号和品牌属性)

#if defined(XUNHU_QTY_MODEL_SETTING)

static char nv_string[32];

char* read_string_from_nv(int flag)

{

int fd,n;

char *buffer = NULL;

int retry=0;

char PN[128];//Length must be no shorter than the path below!!!

sprintf(PN,"/dev/block/platform/mtk-msdc.0/11230000.msdc0/by-name/proinfo");//path may vary from case to case

fd= open(PN,O_RDONLY);

while ((retry < 3)&&(fd < 0)){

sleep(10);

retry++;

fd= open(PN,O_RDONLY);

}

if (fd < 0)

{

return nv_string;

}

buffer = (char*)malloc(2048);

if(buffer == NULL)

{

close(fd);

return nv_string;

}

memset(buffer,0,2048);

n=read(fd,buffer,2048);

if(n != 2048){

close(fd);

return nv_string;

}

close(fd);

if (flag == 0) {

strncpy(nv_string, &buffer[1024 + 570], 32);

} else if (flag == 1) {

strncpy(nv_string, &buffer[1024 + 602], 32);

}

ERROR("qty, initModelAndBrand=>nv_string: %s.", nv_string);

free(buffer);

return nv_string;

}

#endif

///&&}}

（2）在设置作假型号属性值：

static void load_properties(char *data, const char *filter)

{

...

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(更新型号和品牌属性)

//property_set(key, value);

#if defined(XUNHU_QTY_MODEL_SETTING)

if (strcmp(key,"persist.sys.xunhu_model") == 0) {

strcpy(nv_string ,read_string_from_nv(0));

if (strlen(nv_string) > 0) {

property_set(key, nv_string);

} else {

property_set(key, value);

}

} else if (strcmp(key, "persist.sys.xunhu_brand") == 0) {

strcpy(nv_string ,read_string_from_nv(1));

if (strlen(nv_string) > 0) {

property_set(key, nv_string);

} else {

property_set(key, value);

}

} else {

property_set(key, value);

}

#else

property_set(key, value);

#endif

///&&}}

...

}

(3)在system/core/libcutils/properties.c中替换掉ro.product.model和ro.product.brand属性的值：

int property_get(const char *key, char *value, const char *default_value)

{

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(修改品牌和型号)

/*

int len;

len = __system_property_get(key, value);

*/

int len = 0;

#if defined(XUNHU_QTY_MODEL_SETTING)

if (strcmp(key,"ro.product.model") == 0) {

len = __system_property_get("persist.sys.xunhu_model", value);

} else if (strcmp(key, "ro.product.brand") == 0) {

len = __system_property_get("persist.sys.xunhu_brand", value);

}  else if (strcmp(key, "ro.product.model_origin") == 0) {

len = __system_property_get("ro.product.model", value);

} else if (strcmp(key, "ro.product.brand_origin") == 0) {

len = __system_property_get("ro.product.brand", value);

}

ALOGV("mbbwaumpd, property_get(top)=>key:  %s, value: %s, len: %d", key, value, len);

if (len > 0 && strcmp(value, "unknown") != 0) {

return len;

} else {

len = __system_property_get(key, value);

}

ALOGV("mbbwaumpd, property_get(bottom)=>key:  %s, value: %s, len: %d", key, value, len);

#else

len = __system_property_get(key, value);

#endif

///&&}}

if(len > 0) {

return len;

}

if(default_value) {

len = strlen(default_value);

if (len >= PROPERTY_VALUE_MAX) {

len = PROPERTY_VALUE_MAX - 1;

}

memcpy(value, default_value, len);

value[len] = '\0';

}

return len;

}

3. 客制化蓝牙名称：

修改system/bt/btif/src/btif_dm.c文件：

void btif_dm_read_energy_info()

{

… …

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(修改蓝牙名称)

#if defined(XUNHU_QTY_CUSTOM_DEVICES_MODEL_NAME)

char prop_model[PROPERTY_VALUE_MAX];

int max_len = sizeof(btif_default_local_name) - 1;

property_get("persist.sys.custom_dev_model", prop_model, "");

BTIF_TRACE_DEBUG("cdmn, prop_model: %s, length: %d", prop_model, strlen(prop_model));

if (strlen(prop_model) > 0) {

strncpy(btif_default_local_name, prop_model, max_len);

btif_default_local_name[max_len] = '\0';

}

#elif defined(XUNHU_QTY_MODEL_SETTING)

char bt_name[PROPERTY_VALUE_MAX];

int max_len = sizeof(btif_default_local_name) - 1;

property_get("persist.sys.xunhu_bt", bt_name, "");

BTIF_TRACE_DEBUG("mbbwaumpd, bt_name: %s, length: %d", bt_name, strlen(bt_name));

if (strlen(bt_name) > 0) {

strncpy(btif_default_local_name, bt_name, max_len);

btif_default_local_name[max_len] = '\0';

}

#endif

///&&}}

BTIF_TRACE_DEBUG("%s: default name=%s", __FUNCTION__, btif_default_local_name);

return btif_default_local_name;

}

4.客制化WIFI热点名称：

（1）修改frameworks/opt/net/wifi/service/java/com/android/server/wifi/WifiApConfigStore.java文件：

private void setDefaultApConfiguration() {

WifiConfiguration config = new WifiConfiguration();

IWifiFwkExt wifiFwkExt = MPlugin.createInstance(IWifiFwkExt.class.getName(), mContext);

if (SystemProperties.get("ro.mtk_bsp_package").equals("1")) {

if (wifiFwkExt != null) {

config.SSID = wifiFwkExt.getApDefaultSsid();

} else {

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(修改WIFI热点名称)

//config.SSID = mContext.getString(R.string.wifi_tether_configure_ssid_default);

if (XunhuOption.XUNHU_QTY_CUSTOM_DEVICES_MODEL_NAME) {

String ssid = SystemProperties.get("persist.sys.custom_dev_model");

if (!TextUtils.isEmpty(ssid)) {

config.SSID = ssid;

} else {

config.SSID = mContext.getString(R.string.wifi_tether_configure_ssid_default);

}

Log.d("cdmn", "[WifiApConfigStore]setDefaultApConfiguration(1)=>ssid: " + ssid + " config: " + config.SSID);

} else if (XunhuOption.XUNHU_QTY_MODEL_SETTING) {

String ssid = SystemProperties.get("persist.sys.xunhu_wifiap");

if (!TextUtils.isEmpty(ssid)) {

config.SSID = ssid;

} else {

config.SSID = mContext.getString(R.string.wifi_tether_configure_ssid_default);

}

Log.d("mbbwaumpd", "[WifiApConfigStore]setDefaultApConfiguration(1)=>ssid: " + ssid + " config: " + config.SSID);

} else {

config.SSID = mContext.getString(R.string.wifi_tether_configure_ssid_default);

}

}

///&&}}

} else {

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(修改WIFI热点名称)

/*

config.SSID = com.mediatek.custom.CustomProperties.getString(

com.mediatek.custom.CustomProperties.MODULE_WLAN,

com.mediatek.custom.CustomProperties.SSID,

mContext.getString(R.string.wifi_tether_configure_ssid_default));

*/

if (XunhuOption.XUNHU_QTY_CUSTOM_DEVICES_MODEL_NAME) {

String ssid = SystemProperties.get("persist.sys.custom_dev_model");

if (!TextUtils.isEmpty(ssid)) {

config.SSID = ssid;

} else {

config.SSID = com.mediatek.custom.CustomProperties.getString(

com.mediatek.custom.CustomProperties.MODULE_WLAN,

com.mediatek.custom.CustomProperties.SSID,

mContext.getString(R.string.wifi_tether_configure_ssid_default));

}

Log.d("cdmn", "[WifiApConfigStore]setDefaultApConfiguration(2)=>ssid: " + ssid + " config: " + config.SSID);

} else if (XunhuOption.XUNHU_QTY_MODEL_SETTING) {

String ssid = SystemProperties.get("persist.sys.xunhu_wifiap");

if (!TextUtils.isEmpty(ssid)) {

config.SSID = ssid;

} else {

config.SSID = com.mediatek.custom.CustomProperties.getString(

com.mediatek.custom.CustomProperties.MODULE_WLAN,

com.mediatek.custom.CustomProperties.SSID,

mContext.getString(R.string.wifi_tether_configure_ssid_default));

}

Log.d("mbbwaumpd", "[WifiApConfigStore]setDefaultApConfiguration(2)=>ssid: " + ssid + " config: " + config.SSID);

} else {

config.SSID = com.mediatek.custom.CustomProperties.getString(

com.mediatek.custom.CustomProperties.MODULE_WLAN,

com.mediatek.custom.CustomProperties.SSID,

mContext.getString(R.string.wifi_tether_configure_ssid_default));

}

///&&}}

if (wifiFwkExt != null && wifiFwkExt.needRandomSsid()) {

Random random = new Random(SystemClock.elapsedRealtime());

config.SSID = config.SSID + random.nextInt(1000);

Log.d(TAG, "setDefaultApConfiguration, SSID:" + config.SSID);

}

}

config.allowedKeyManagement.set(KeyMgmt.WPA2_PSK);

String randomUUID = UUID.randomUUID().toString();

//first 12 chars from xxxxxxxx-xxxx-4xxx-yxxx-xxxxxxxxxxxx

config.preSharedKey = randomUUID.substring(0, 8) + randomUUID.substring(9,13);

sendMessage(WifiStateMachine.CMD_SET_AP_CONFIG, config);

}

（2）以及替换掉所有使用 R.string.wifi_tether_configure_ssid_default字符串的地方，主要是Settings应用。

4. 客制化MTP名称：

修改frameworks/base/media/java/android/mtp/MtpDatabase.java文件：

private int getDeviceProperty(int property, long[] outIntValue, char[] outStringValue) {

Log.d(TAG, "getDeviceProperty  property = 0x" + Integer.toHexString(property));

switch (property) {

case MtpConstants.DEVICE_PROPERTY_SYNCHRONIZATION_PARTNER:

case MtpConstants.DEVICE_PROPERTY_DEVICE_FRIENDLY_NAME:

// writable string properties kept in shared preferences

String value = mDeviceProperties.getString(Integer.toString(property), "");

int length = value.length();

if (length > 255) {

length = 255;

}

value.getChars(0, length, outStringValue, 0);

outStringValue[length] = 0;

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(修改MTP在PC"我的电脑"中显示的label名称)

if (XunhuOption.XUNHU_QTY_CUSTOM_DEVICES_MODEL_NAME) {

Log.d("cdmn", "[MtpDatabase]getDeviceProperty=>value: " + value + " length: " + length);

String deviceName = SystemProperties.get("persist.sys.custom_dev_model");

if (!TextUtils.isEmpty(deviceName)) {

int lengthDeviceName = deviceName.length();

if (lengthDeviceName > 255) {

lengthDeviceName = 255;

}

if(lengthDeviceName > 0) {

deviceName.getChars(0, lengthDeviceName, outStringValue, 0);

outStringValue[lengthDeviceName] = 0;

Log.d("cdmn", "[MtpDatabase]getDeviceProperty=>deviceName = " + deviceName + ", lengthDeviceName = " + lengthDeviceName);

} else {

Log.d("cdmn", "[MtpDatabase]getDeviceProperty=>lengthDeviceName = " + lengthDeviceName);

}

} else {

deviceName = SystemProperties.get("ro.product.model", "");

int lengthDeviceName = deviceName.length();

if (lengthDeviceName > 255) {

lengthDeviceName = 255;

}

if(lengthDeviceName > 0) {

deviceName.getChars(0, lengthDeviceName, outStringValue, 0);

outStringValue[lengthDeviceName] = 0;

Log.d("cdmn", "[MtpDatabase]getDeviceProperty=>deviceName = " + deviceName + ", lengthDeviceName = " + lengthDeviceName);

} else {

Log.d("cdmn", "[MtpDatabase]getDeviceProperty=>lengthDeviceName = " + lengthDeviceName);

}

}

} else if (XunhuOption.XUNHU_QTY_MODEL_SETTING) {

String usbConfig = SystemProperties.get("sys.usb.config");

String mtpName = SystemProperties.get("persist.sys.xunhu_mtp");

if (!TextUtils.isEmpty(usbConfig) && usbConfig.contains("ptp")) {

mtpName = SystemProperties.get("persist.sys.xunhu_ptp");

}

if (!TextUtils.isEmpty(mtpName)) {

int nameLength = mtpName.length();

if (nameLength > 255) {

nameLength = 255;

}

mtpName.getChars(0, nameLength, outStringValue, 0);

outStringValue[nameLength] = 0;

Log.d("mbbwaumpd", "[MtpDatabase]getDeviceProperty=>mtpName = " + mtpName + ", nameLength = " + nameLength);

}

}

///&&}

return MtpConstants.RESPONSE_OK;

case MtpConstants.DEVICE_PROPERTY_IMAGE_SIZE:

// use screen size as max image size

Display display = ((WindowManager)mContext.getSystemService(

Context.WINDOW_SERVICE)).getDefaultDisplay();

int width = display.getMaximumSizeDimension();

int height = display.getMaximumSizeDimension();

String imageSize = Integer.toString(width) + "x" +  Integer.toString(height);

imageSize.getChars(0, imageSize.length(), outStringValue, 0);

outStringValue[imageSize.length()] = 0;

return MtpConstants.RESPONSE_OK;

// DEVICE_PROPERTY_BATTERY_LEVEL is implemented in the JNI code

default:

return MtpConstants.RESPONSE_DEVICE_PROP_NOT_SUPPORTED;

}

}

5. 客制化PTP名称：

修改frameworks/av/media/mtp/MtpServer.cpp文件：

MtpResponseCode MtpServer::doGetDeviceInfo() {

MtpStringBuffer   string;

char prop_value[PROPERTY_VALUE_MAX];

MtpObjectFormatList* playbackFormats = mDatabase->getSupportedPlaybackFormats();

MtpObjectFormatList* captureFormats = mDatabase->getSupportedCaptureFormats();

MtpDevicePropertyList* deviceProperties = mDatabase->getSupportedDeviceProperties();

// fill in device info

mData.putUInt16(MTP_STANDARD_VERSION);

if (mPtp) {

mData.putUInt32(0);

} else {

// MTP Vendor Extension ID

mData.putUInt32(6);

}

mData.putUInt16(MTP_STANDARD_VERSION);

if (mPtp) {

// no extensions

string.set("");

} else {

// MTP extensions

string.set("microsoft.com: 1.0; android.com: 1.0;");

}

mData.putString(string); // MTP Extensions

mData.putUInt16(0); //Functional Mode

mData.putAUInt16(kSupportedOperationCodes,

sizeof(kSupportedOperationCodes) / sizeof(uint16_t)); // Operations Supported

mData.putAUInt16(kSupportedEventCodes,

sizeof(kSupportedEventCodes) / sizeof(uint16_t)); // Events Supported

mData.putAUInt16(deviceProperties); // Device Properties Supported

mData.putAUInt16(captureFormats); // Capture Formats

mData.putAUInt16(playbackFormats);  // Playback Formats

property_get("ro.product.manufacturer", prop_value, "unknown manufacturer");

string.set(prop_value);

mData.putString(string);   // Manufacturer

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(PTP在PC"我的电脑"中显示的label名称)

//property_get("ro.product.model", prop_value, "MTP Device");

#if defined(XUNHU_QTY_CUSTOM_DEVICES_MODEL_NAME)

property_get("persist.sys.custom_dev_model", prop_value, "MTP Device");

ALOGE("cdmn, [MtpServer]doGetDeviceInfo=>prop_value length: %d", strlen(prop_value));

if(strlen(prop_value) <= 0) {

property_get("ro.product.model", prop_value, "MTP Device");

}

#elif defined(XUNHU_QTY_MODEL_SETTING)

char usb_config[PROPERTY_VALUE_MAX];

property_get("sys.usb.config", usb_config, "");

if (strlen(usb_config) > 0 && strstr(usb_config, "mtp")) {

property_get("persist.sys.xunhu_mtp", prop_value, "");

} else {

property_get("persist.sys.xunhu_ptp", prop_value, "");

}

ALOGE("mbbwaumpd, [MtpServer]doGetDeviceInfo=>prop_value length: %d", strlen(prop_value));

if(strlen(prop_value) <= 0) {

property_get("ro.product.model", prop_value, "MTP Device");

}

#else

property_get("ro.product.model", prop_value, "MTP Device");

#endif

ALOGE("mbbwaumpd, [MtpServer]doGetDeviceInfo=>prop_value: %s", prop_value);

///&&}}

string.set(prop_value);

mData.putString(string);   // Model

string.set("1.0");

mData.putString(string);   // Device Version

property_get("ro.serialno", prop_value, "????????");

string.set(prop_value);

mData.putString(string);   // Serial Number

delete playbackFormats;

delete captureFormats;

delete deviceProperties;

return MTP_RESPONSE_OK;

}

7. 客制化USB和资源管理器磁盘名称：

公共部分：

（1）将usb名称和资源管理器磁盘名称写入/proc/cmdline节点中：

a. 在/vendor/mediatek/proprietary/bootloader/lk/platform/mt6735/load_image.c文件中添加如下代码：

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(修改USB和资源管理器名称)

#ifdef XUNHU_QTY_MODEL_SETTING

static char manufacturer_temp[256];

static char product_temp[256];

static char usb_dm[32];

// flags => 0 : usb 1: devices manager

char* get_name_from_nv(int flags) {

char part_name[]="proinfo";

unsigned char addr[2048]={0};

long len;

#ifdef MTK_EMMC_SUPPORT

unsigned long long start_addr;

#else

unsigned long start_addr;

#endif

part_t *part;

part_dev_t *dev;

dev = mt_part_get_device();

if (!dev)

{

return 0;

}

part = mt_part_get_partition(part_name);

if (!part)

{

return 0;

}

#ifdef MTK_EMMC_SUPPORT

start_addr = (u64)part->start_sect * BLK_SIZE;

#else

start_addr = part->startblk * BLK_SIZE;

#endif

#ifdef MTK_EMMC_SUPPORT

#ifdef MTK_NEW_COMBO_EMMC_SUPPORT

len = dev->read(dev, start_addr, (uchar*)addr, 2048, part->part_id);

#else

len = dev->read(dev, start_addr, (uchar*)addr, 2048);

#endif

#else

len = dev->read(dev, start_addr, (uchar*)addr, 2048);

#endif

if (len < 0)

{

return 0;

}

if (flags == 0) {

strncpy(usb_dm, &addr[1024+698], 32);

} else if (flags == 1) {

strncpy(usb_dm, &addr[1024+796], 32);

} else {

return 0;

}

return  usb_dm;

}

#endif

///&&}}

b. 修改/vendor/mediatek/proprietary/bootable/bootloader/lk/app/mt_boot/mt_boot.c文件：

添加如下代码：

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(修改USB和资源管理器名称)

#ifdef XUNHU_QTY_MODEL_SETTING

extern char* get_name_from_nv(int);

#endif

///&&}}

修改如下函数：

int boot_linux_from_storage(void)

{

…...

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(修改USB和资源管理器名称)

#ifdef XUNHU_QTY_MODEL_SETTING

char temp[42];

snprintf(temp, 42, "usb=%s-usb", get_name_from_nv(0));

cmdline_append(temp);

snprintf(temp, 42, "dm=%s-dm", get_name_from_nv(1));

cmdline_append(temp);

#endif

///&&}}

…...

}

(2)修改build/core/Makefile文件：

#Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

#Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称

$(hide) CUSTOM_MODEM="$(CUSTOM_MODEM)" \

XUNHU_QTY_MODEL="$(XUNHU_QTY_MODEL)" \

XUNHU_QTY_BRAND="$(XUNHU_QTY_BRAND)" \

XUNHU_QTY_BT="$(XUNHU_QTY_BT)" \

XUNHU_QTY_WIFI_AP="$(XUNHU_QTY_WIFI_AP)" \

XUNHU_QTY_USB="$(XUNHU_QTY_USB)" \

XUNHU_QTY_MTP="$(XUNHU_QTY_MTP)" \

XUNHU_QTY_PTP="$(XUNHU_QTY_PTP)" \

XUNHU_QTY_DM="$(XUNHU_QTY_DM)" \

bash $(GITINFO_SH) $@ >> $@

#xunhu:add by lww end

（3）在/build/tools/gitinfo.sh文件中添加如下代码：

#Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

#Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称

if [[ "$XUNHU_QTY_MODEL" != "no" ]] ; then

echo "persist.sys.xunhu_model=$XUNHU_QTY_MODEL"

echo "persist.sys.xunhu_model_b=$XUNHU_QTY_MODEL"

fi

if [[ "$XUNHU_QTY_BRAND" != "no" ]] ; then

echo "persist.sys.xunhu_brand=$XUNHU_QTY_BRAND"

echo "persist.sys.xunhu_brand_b=$XUNHU_QTY_BRAND"

fi

if [[ "$XUNHU_QTY_BT" != "no" ]] ; then

echo "persist.sys.xunhu_bt=$XUNHU_QTY_BT"

echo "persist.sys.xunhu_bt_b=$XUNHU_QTY_BT"

fi

if [[ "$XUNHU_QTY_WIFI_AP" != "no" ]] ; then

echo "persist.sys.xunhu_wifiap=$XUNHU_QTY_WIFI_AP"

echo "persist.sys.xunhu_wifiap_b=$XUNHU_QTY_WIFI_AP"

fi

if [[ "$XUNHU_QTY_USB" != "no" ]] ; then

echo "persist.sys.xunhu_usb=$XUNHU_QTY_USB"

echo "persist.sys.xunhu_usb_b=$XUNHU_QTY_USB"

fi

if [[ "$XUNHU_QTY_MTP" != "no" ]] ; then

echo "persist.sys.xunhu_mtp=$XUNHU_QTY_MTP"

echo "persist.sys.xunhu_mtp_b=$XUNHU_QTY_MTP"

fi

if [[ "$XUNHU_QTY_PTP" != "no" ]] ; then

echo "persist.sys.xunhu_ptp=$XUNHU_QTY_PTP"

echo "persist.sys.xunhu_ptp_b=$XUNHU_QTY_PTP"

fi

if [[ "$XUNHU_QTY_DM" != "no" ]] ; then

echo "persist.sys.xunhu_dm=$XUNHU_QTY_DM"

echo "persist.sys.xunhu_dm_b=$XUNHU_QTY_DM"

fi

#&&}}

（4）在/kernel-3.18/Makefile文件中添加如下代码：

#Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

#Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称

USERINCLUDE += -I$(srctree)/../out/Autogen_out/custom_model

#&&}}

USB名称部分：

（1）修改/kernel-3.18/drivers/usb/gadget/android.c文件：

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称

#include "teksun_custom_model.h"

#include <linux/string.h>

#include <linux/ctype.h>

///&&}}

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(修改USB名称)

#ifdef XUNHU_QTY_MODEL_SETTING

extern char * saved_command_line;

extern int is_space_string(char*);

static char usb_name[32];

static char manufacturer_temp[256];

static char product_temp[256];

char* get_usb_name_from_cmdline(void) {

int i;

int start_index = -1;

int end_index = -1;

for (i=0; i< strlen(saved_command_line); i++) {

if (strncmp(saved_command_line+i, "usb=", strlen("usb="))==0) {

start_index = i;

}

if (strncmp(saved_command_line+i, "-usb", strlen("-usb"))==0) {

end_index = i;

break;

}

}

pr_info("mbbwaumpd, [android.c]get_usb_name_from_cmdline=>cmdline: %s\n", saved_command_line);

pr_info("mbbwaumpd, [android.c]get_usb_name_from_cmdline=>start: %d, end: %d\n", start_index, end_index);

if (start_index != -1 && end_index != -1 && start_index + strlen("usb=") != end_index) {

strncpy(usb_name, saved_command_line + (start_index + strlen("usb=")), end_index - (start_index + strlen("usb=")));

pr_info("mbbwaumpd, [android.c]get_usb_name_from_cmdline=>is_space_str: %d.\n", is_space_string(usb_name));

if (is_space_string(usb_name) == 1) {

memset(usb_name, 0,sizeof(usb_name));

}

}  else {

#ifdef XUNHU_QTY_USB

strncpy(usb_name, XUNHU_QTY_USB, sizeof(usb_name) - 1);

#else

memset(usb_name, 0,sizeof(usb_name));

#endif

}

return usb_name;

}

#endif

///&&}}

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(修改USB名称)

#if defined(XUNHU_QTY_CUSTOM_DEVICES_MODEL_NAME)

DESCRIPTOR_STRING_ATTR(iManufacturer, product_string_temp)

DESCRIPTOR_STRING_ATTR(iProduct, product_string_temp)

#elif defined(XUNHU_QTY_MODEL_SETTING)

DESCRIPTOR_STRING_ATTR(iManufacturer, manufacturer_temp)

DESCRIPTOR_STRING_ATTR(iProduct, product_temp)

#else

DESCRIPTOR_STRING_ATTR(iManufacturer, manufacturer_string)

DESCRIPTOR_STRING_ATTR(iProduct, product_string)

#endif

///&&}}

static int android_bind(struct usb_composite_dev *cdev)

{

…...

/* Default strings - should be updated by userspace */

///Xunhu: custom model, brand, bt, wifi ap, usb, mtp, ptp, devices manager at 2016-10-28 12:17:12 by qty{{&&

///Description: 客制化型号、品牌、蓝牙、wifi热点、usb、mtp、ptp和devices manager名称(修改USB名称)

/*

strncpy(manufacturer_string, MANUFACTURER_STRING, sizeof(manufacturer_string) - 1);

strncpy(product_string, PRODUCT_STRING, sizeof(product_string) - 1);

*/

#ifdef XUNHU_QTY_MODEL_SETTING

get_usb_name_from_cmdline();

if (strlen(usb_name) > 0) {

memset(manufacturer_string,0,sizeof(manufacturer_string));

strncpy(product_string, usb_name, sizeof(product_string) - 1);

} else {

if (strlen(manufacturer_temp) > 0) {

strncpy(manufacturer_string, manufacturer_temp, sizeof(manufacturer_string) - 1);

} else {

strncpy(manufacturer_string, MANUFACTURER_STRING, sizeof(manufacturer_string) - 1);

}

if (strlen(product_temp) > 0) {

strncpy(product_string, product_temp, sizeof(product_string) - 1);

} else {

strncpy(product_string, PRODUCT_STRING, sizeof(product_string) - 1);

}

}

#else

strncpy(manufacturer_string, MANUFACTURER_STRING, sizeof(manufacturer_string) - 1);

strncpy(product_string, PRODUCT_STRING, sizeof(product_string) - 1);

#endif

///&&}}

…...

}